Module 17 – Electric Circuits & Electrical Safety

| Name: | Class: |
| --- | --- |

17.1	Circuit Diagrams & Components

At the end of this section, students should be able to:

Draw and interpret circuit diagrams using standard circuit symbols.

Describe the function of common circuit components.

Describe how thermistors and LDRs behave in circuits.

[Supplement] Explain how diodes and LEDs work in circuits.

17.1.1	Circuit Symbols

Introduction

Circuit diagrams use standardised symbols to represent electrical components. Learning these symbols is essential for understanding and drawing circuits.

| Component | Description |
| --- | --- |
| Cell | Single source of e.m.f. (long line = +, short = -) |
| Battery | Two or more cells connected in series |
| Power supply | Mains-powered source of electrical energy |
| Generator | Converts mechanical energy to electrical energy |
| Switch (open) | Gap in circuit – no current flows |
| Switch (closed) | Circuit complete – current flows |
| Fixed resistor | Component with constant resistance |
| Variable resistor | Resistor whose resistance can be changed |
| Thermistor (NTC) | Resistance decreases as temperature increases |
| LDR | Resistance decreases as light intensity increases |
| Heater | Transfers electrical energy to thermal energy |
| Lamp | Emits light when current flows |
| Motor | Rotates when current flows |
| Bell | Emits sound when current flows |
| Ammeter | Measures current (connected in SERIES) |
| Voltmeter | Measures p.d. (connected in PARALLEL) |
| Fuse | Melts if current exceeds rated value |
| Relay | Uses small current to switch larger current |
| Transformer | Changes voltage using electromagnetic induction |
| Magnetising coil | Creates magnetic field when current flows |

Important: Ammeters are ALWAYS connected in SERIES with the component.

Important: Voltmeters are ALWAYS connected in PARALLEL across the component.

17.1.2	Thermistors

Definition
A thermistor is a resistor whose resistance changes with temperature.

For an NTC (Negative Temperature Coefficient) thermistor:

As TEMPERATURE INCREASES → RESISTANCE DECREASES

As TEMPERATURE DECREASES → RESISTANCE INCREASES

This property makes thermistors useful in temperature-sensing circuits, such as:

Thermostats

Fire alarms

Temperature-controlled fans

17.1.3	Light-Dependent Resistors (LDRs)

Definition
A light-dependent resistor (LDR) is a resistor whose resistance changes depending on the light intensity falling on it.

For an LDR:

As LIGHT INTENSITY INCREASES → RESISTANCE DECREASES

As LIGHT INTENSITY DECREASES → RESISTANCE INCREASES

This property makes LDRs useful in light-sensing circuits, such as:

Automatic street lights

Camera light meters

Burglar alarms

17.1.4	Diodes and LEDs [Supplement]

[Extended Tier Only]

Definition
A diode is a component that allows current to flow in one direction only.

Key properties of diodes:

Current can only flow in the direction the triangle points (forward bias).

If the diode is reversed, no current flows (reverse bias).

A light-emitting diode (LED) emits light when current flows through it.

Check Your Understanding: Circuit Components

Q1. State one difference between an ammeter and a voltmeter in terms of how they are connected in a circuit. [1]

…………………………………………………….

…………………………………………………….

Q2. Describe what happens to the resistance of a thermistor when it is heated. [1]

…………………………………………………….

…………………………………………………….

Q3. A street lamp is designed to switch on automatically when it gets dark. Explain which component would be suitable for detecting the light level. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

Q4. [Supplement] Explain why a diode is sometimes called a "one-way valve" for electricity. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

17.2	Series Circuits

At the end of this section, students should be able to:

Know that current at every point in a series circuit is the same.

Calculate the combined e.m.f. of several sources in series.

[Supplement] Know that total p.d. in series equals sum of individual p.d.s.

Calculate combined resistance of resistors in series.

17.2.1	What is a Series Circuit?

Definition
A series circuit is a circuit with only one path for current to flow.

In a series circuit:

All components are connected in a single loop.

The same current flows through every component.

If one component breaks, the whole circuit stops working.

17.2.2	Current in Series Circuits

Key Rule
In a series circuit, the current is the SAME at every point.

Why is this true?

Current is the flow of electrons.

Electrons cannot be created or destroyed in the circuit.

The same number of electrons per second passes through each component.

Formula:  I₁ = I₂ = I₃ = … = I (everywhere in the circuit)

Factors affecting current in a series circuit:

1. VOLTAGE of power source: Higher voltage → more current

2. RESISTANCE in circuit: More components → more resistance → less current

17.2.3	E.M.F. in Series

When cells or batteries are connected in series:

Key Rule
The combined e.m.f. equals the SUM of individual e.m.f.s.

Formula:  E_total = E₁ + E₂ + E₃ + …

Example: If three 1.5 V cells are connected in series:

E_total = 1.5 + 1.5 + 1.5 = 4.5 V

17.2.4	Potential Difference in Series [Supplement]

[Extended Tier Only]

Key Rule
In a series circuit, the sum of p.d.s across all components equals the total e.m.f. of the power source.

Formula:  E = V₁ + V₂ + V₃ + …

where E = e.m.f. of power source (in V), V₁, V₂, V₃ = p.d. across each component (in V)

This means the e.m.f. is "shared" between the components in series.

Worked Example 1: Finding Missing P.D.

Problem
A power source has an e.m.f. of 12 V. In a series circuit, the p.d. across resistor X is 8 V. Calculate the p.d. across resistor Y.

Solution
Step 1: Identify given values

Given: E = 12 V, V_X = 8 V

Required: V_Y = ?

Step 2: Recall the rule for p.d. in series

E = V_X + V_Y

Step 3: Rearrange and substitute

V_Y = E – V_X = 12 – 8 = 4 V

Answer: The p.d. across resistor Y is 4 V.

17.2.5	Resistance in Series

Key Rule
The combined resistance of resistors in series equals the SUM of the individual resistances.

Formula:  R_total = R₁ + R₂ + R₃ + …

where R_total = total resistance (in Ω), R₁, R₂, R₃ = resistance of each resistor (in Ω)

Worked Example 2: Combined Resistance in Series

Problem
Three resistors of 4 Ω, 6 Ω and 10 Ω are connected in series. Calculate the total resistance.

Solution
Step 1: Identify given values

R₁ = 4 Ω, R₂ = 6 Ω, R₃ = 10 Ω

Step 2: Apply the formula for resistors in series

R_total = R₁ + R₂ + R₃ = 4 + 6 + 10 = 20 Ω

Answer: The total resistance is 20 Ω.

Check Your Understanding: Series Circuits

Q5. State what happens to the current at different points in a series circuit. [1]

…………………………………………………….

…………………………………………………….

Q6. Three 1.5 V cells are connected in series. Calculate the total e.m.f. [1]

…………………………………………………….

…………………………………………………….

Q7. [Supplement] A 9 V battery is connected to two resistors in series. If the p.d. across one resistor is 3 V, what is the p.d. across the other? [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

Q8. Calculate the total resistance when a 5 Ω and 15 Ω resistor are connected in series. [1]

…………………………………………………….

…………………………………………………….

17.3	Parallel Circuits

At the end of this section, students should be able to:

Know how to construct and use parallel circuits.

State that current from the source is larger than current in each branch.

[Supplement] Recall and use the junction rule for currents.

[Supplement] Know that p.d. across parallel branches is the same.

Calculate combined resistance of resistors in parallel.

State advantages of connecting lamps in parallel.

17.3.1	What is a Parallel Circuit?

Definition
A parallel circuit is a circuit with multiple paths (branches) for current to flow.

In a parallel circuit:

Components are connected in separate branches.

Current splits at junctions and recombines.

If one branch breaks, current can still flow through other branches.

17.3.2	Current in Parallel Circuits

Key Rule
The current from the source is LARGER than the current in each branch.

At a junction (where wires meet):

Current SPLITS into different branches.

Current RECOMBINES when branches join.

Junction Rule [Supplement]

[Extended Tier Only]

Key Rule
The sum of currents entering a junction equals the sum of currents leaving the junction.

Formula:  I_in = I_out    or    I_total = I₁ + I₂ + I₃ + …

This is because charge is conserved – electrons cannot be created or destroyed.

Worked Example 3: Current at a Junction

Problem
In a parallel circuit, ammeter A₀ shows a reading of 10 A, and ammeter A₁ shows 6 A. Calculate the reading on ammeter A₂.

Solution
Step 1: Recall the junction rule

The total current entering a junction equals the total current leaving.

Step 2: Apply the junction rule

I₀ = I₁ + I₂

I₂ = I₀ – I₁ = 10 – 6 = 4 A

Answer: The reading on ammeter A₂ is 4 A.

17.3.3	Potential Difference in Parallel [Supplement]

[Extended Tier Only]

Key Rule
The p.d. across each branch in a parallel circuit is the SAME.

Formula:  V₁ = V₂ = V₃ = … = E

This means each branch receives the full voltage of the power supply.

17.3.4	Resistance in Parallel

Key Rule
The combined resistance of two resistors in parallel is LESS than the resistance of either resistor alone.

Why is this true?

In parallel, there are more paths for current to flow.

More paths means less overall resistance to current.

Special Case: Two identical resistors in parallel → Combined resistance = half of one resistor.

Example: Two 4 Ω resistors in parallel → R_total = 2 Ω

Calculating Resistance in Parallel [Supplement]

[Extended Tier Only]

Formula:  1/R_total = 1/R₁ + 1/R₂

Steps to calculate:

1. Calculate 1/R₁ and 1/R₂

2. Add them together to get 1/R_total

3. Take the reciprocal to find R_total

Worked Example 4: Resistance in Parallel [Supplement]

Problem
A 6 Ω resistor and a 3 Ω resistor are connected in parallel. Calculate the combined resistance.

Solution
Step 1: Identify given values

R₁ = 6 Ω, R₂ = 3 Ω

Step 2: Apply the formula

1/R_total = 1/R₁ + 1/R₂ = 1/6 + 1/3 = 1/6 + 2/6 = 3/6

Step 3: Take the reciprocal

R_total = 6/3 = 2 Ω

Answer: The combined resistance is 2 Ω. Note: The combined resistance is less than either individual resistor.

17.3.5	Advantages of Parallel Lighting Circuits

Lighting circuits in homes and offices use lamps connected in PARALLEL.

| Advantage | Explanation |
| --- | --- |
| Same brightness | Each lamp receives the full voltage |
| Independent operation | If one lamp breaks, others still work |
| Individual control | Each lamp can be switched on/off separately |

Check Your Understanding: Parallel Circuits

Q9. State what happens to current at a junction in a parallel circuit. [1]

…………………………………………………….

…………………………………………………….

Q10. [Supplement] The total current from a battery is 8 A. If 3 A flows through one branch, how much flows through the other? [1]

…………………………………………………….

…………………………………………………….

Q11. State one advantage of connecting lamps in parallel rather than series. [1]

…………………………………………………….

…………………………………………………….

Q12. Two 10 Ω resistors are connected in parallel. State whether the combined resistance is greater than, equal to, or less than 10 Ω. [1]

…………………………………………………….

…………………………………………………….

Q13. [Supplement] Calculate the combined resistance of a 4 Ω and 12 Ω resistor in parallel. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

17.4	Potential Dividers

At the end of this section, students should be able to:

Know that p.d. across a conductor increases as resistance increases.

[Supplement] Describe the action of a variable potential divider.

[Supplement] Recall and use the equation R₁/R₂ = V₁/V₂.

17.4.1	Relationship between P.D. and Resistance

When current flows through resistors in series:

Key Rule
For a constant current, the p.d. across a component increases as its resistance increases.

This follows from Ohm’s law: V = IR

If I is constant and R increases, V must increase.

The component with the higher resistance has the larger p.d.

17.4.2	Variable Potential Dividers [Supplement]

[Extended Tier Only]

Definition
A potential divider is a circuit that uses resistors in series to split the voltage from a power source.

A potential divider allows you to obtain a fraction of the total voltage.

How it works:

Two resistors are connected in series.

The p.d. splits between them in the same ratio as their resistances.

A voltmeter or component connected in parallel measures/uses part of the voltage.

If one resistor is variable (or is a thermistor/LDR):

Changing its resistance changes how the voltage is divided.

This can be used to control other components automatically.

17.4.3	Potential Divider Equation [Supplement]

[Extended Tier Only]

Key Equation:  R₁/R₂ = V₁/V₂

where R₁, R₂ = resistance of each resistor (in Ω), V₁, V₂ = p.d. across each resistor (in V)

The voltage splits in the SAME RATIO as the resistances.

Remember: V₁ + V₂ = E (total e.m.f.)

Worked Example 5: Potential Divider Calculation [Supplement]

Problem
A potential divider has R₁ = 12 kΩ and R₂ = 20 kΩ. The output voltage V_out (across R₂) is 5.0 V. Calculate the input voltage V_in.

Solution
Step 1: Identify given values

R₁ = 12 kΩ, R₂ = 20 kΩ, V₂ = V_out = 5.0 V

Step 2: Use the potential divider equation

R₁/R₂ = V₁/V₂

12/20 = V₁/5.0

V₁ = (12/20) × 5.0 = 3.0 V

Step 3: Calculate total input voltage

V_in = V₁ + V₂ = 3.0 + 5.0 = 8.0 V

Answer: The input voltage is 8.0 V.

Check Your Understanding: Potential Dividers

Q14. In a series circuit with constant current, what happens to the p.d. across a resistor if its resistance doubles? [1]

…………………………………………………….

…………………………………………………….

Q15. [Supplement] A potential divider has a 10 Ω and 30 Ω resistor. If the total voltage is 8 V, calculate the p.d. across each resistor. [3]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

17.5	Electrical Safety

At the end of this section, students should be able to:

State the hazards of damaged insulation, overheating cables, damp conditions, and excess current from overloading.

Know the three wires in a mains circuit and their functions.

Explain why a switch must be connected to the live wire.

Explain the use and operation of fuses and trip switches.

Explain double insulation and earthing.

17.5.1	Electrical Hazards

Mains electricity (typically 230 V in UK/Europe, 120 V in USA) is dangerous.

| Hazard | Risk |
| --- | --- |
| Damaged insulation | Exposed wires can cause electric shock |
| Overheating cables | Excess current causes wires to heat; can cause fire |
| Damp conditions | Water conducts electricity; increases shock risk |
| Overloading | Too many appliances on one socket causes overheating |

17.5.2	Mains Circuits – The Three Wires

A mains circuit consists of THREE wires:

| Wire | Colour | Function |
| --- | --- | --- |
| LIVE | Brown | Carries current to the appliance at mains voltage (230 V). MOST DANGEROUS wire. |
| NEUTRAL | Blue | Completes the circuit. At 0 V (or close to 0 V). |
| EARTH | Green/Yellow stripes | Safety wire. Provides low-resistance path to ground. Only carries current during a fault. |

Important: The switch MUST be connected to the LIVE wire.

Why?

When the switch is off, no voltage reaches the appliance.

This makes it safe to touch or repair the appliance.

If the switch was on the neutral wire, the appliance would still be "live" even when switched off.

17.5.3	Fuses

Definition
A fuse is a safety device containing a thin wire that melts if the current becomes too large, breaking the circuit.

How fuses work:

1. Normal operation: Current flows through the fuse wire.

2. Fault (excess current): Wire heats up and MELTS.

3. Circuit broken: No current flows, appliance is disconnected.

Fuse ratings:

Common values: 3 A, 5 A, 13 A

Choose the rating JUST ABOVE the normal operating current.

Example: An appliance normally uses 2.5 A

3 A fuse: Correct choice (just above 2.5 A)

5 A fuse: Too high (would allow dangerous current before blowing)

13 A fuse: Far too high (could cause fire before blowing)

Key Point: A fuse protects the circuit and cabling, NOT the user. For a double-insulated appliance, the fuse protects the circuit and cables.

17.5.4	Trip Switches (Circuit Breakers)

Definition
A trip switch (circuit breaker) is a safety device that automatically switches off if the current becomes too large.

Advantages of trip switches over fuses:

| Trip Switches | Fuses |
| --- | --- |
| Can be RESET after tripping | Must be REPLACED after blowing |
| Reusable | One-time use |
| React faster | Slower response |

17.5.5	Earthing and Double Insulation

Earthing:

For appliances with METAL cases:

The earth wire connects the metal case to the ground.

If a live wire touches the case:

1. Current flows through earth wire (low resistance path).

2. Large surge of current causes fuse to blow.

3. Circuit is disconnected, preventing electric shock.

Double Insulation:

For appliances with PLASTIC cases (non-conducting outer):

No earth wire is needed.

The plastic case cannot become live.

Two layers of insulation protect the user:

1. Insulation around the wires.

2. Non-conducting outer case.

Key Point: An appliance must be EITHER earthed (metal case connected to earth wire) OR double-insulated (non-conducting outer case).

Check Your Understanding: Electrical Safety

Q16. State two electrical hazards in the home. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

Q17. Name the three wires in a mains circuit and state their colours. [3]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

Q18. Explain why the switch in a circuit must be connected to the live wire. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

Q19. An appliance normally draws 4.5 A. What fuse rating should be used? (Choose from 3 A, 5 A, or 13 A) [1]

…………………………………………………….

…………………………………………………….

Q20. Explain how earthing protects a user from electric shock. [3]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

Q21. Explain why a double-insulated appliance does not need an earth wire. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

Module Summary

Circuit Components:

| Component | Behaviour |
| --- | --- |
| Thermistor | Resistance DECREASES as temperature INCREASES |
| LDR | Resistance DECREASES as light intensity INCREASES |
| Diode | Allows current in ONE direction only |

Series Circuits:

| Property | Rule |
| --- | --- |
| Current | Same at all points: I₁ = I₂ = I₃ |
| E.M.F. | Sum of individual e.m.f.s: E_total = E₁ + E₂ + E₃ |
| P.D. | Sum equals total e.m.f.: E = V₁ + V₂ + V₃ |
| Resistance | Sum of individual values: R_total = R₁ + R₂ + R₃ |

Parallel Circuits:

| Property | Rule |
| --- | --- |
| Current | Splits at junctions: I_total = I₁ + I₂ + I₃ |
| P.D. | Same across each branch: V₁ = V₂ = V₃ = E |
| Resistance | Less than smallest value: 1/R = 1/R₁ + 1/R₂ |

Potential Dividers:

Voltage splits in same ratio as resistance: R₁/R₂ = V₁/V₂

Electrical Safety:

| Feature | Detail |
| --- | --- |
| Live wire | Brown – carries current at mains voltage |
| Neutral wire | Blue – completes circuit at ~0 V |
| Earth wire | Green/Yellow – safety wire to ground |
| Fuse | Melts when current too high, breaks circuit |
| Trip switch | Automatically switches off, can be reset |
| Double insulation | Non-conducting case, no earth needed |

Common Misconceptions

| Misconception | Correction |
| --- | --- |
| "Current is used up in a circuit" | Current is the SAME throughout a series circuit – charge is conserved. |
| "Adding more resistors in parallel increases resistance" | Adding resistors in parallel DECREASES total resistance (more paths for current). |
| "The fuse protects the user" | The fuse protects the circuit and wiring; the earth wire protects the user. |
| "Voltage is split equally in a potential divider" | Voltage splits in the RATIO of resistances (higher resistance = higher p.d.). |

Practice Exercises

Section A: Recall Questions

1. Draw the circuit symbols for: (a) a cell, (b) a lamp, (c) a fuse, (d) a thermistor, (e) an LDR. [5]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

2. State the rule for current in a series circuit. [1]

…………………………………………………….

…………………………………………………….

3. State the rule for current at a junction in a parallel circuit. [1]

…………………………………………………….

…………………………………………………….

4. State the three wires in a mains circuit and their colours. [3]

…………………………………………………….

…………………………………………………….

…………………………………………………….

5. State two advantages of trip switches over fuses. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

Section B: Application Questions

6. Two cells, each with e.m.f. 1.5 V, are connected in series. Calculate the total e.m.f. [1]

…………………………………………………….

…………………………………………………….

7. Three resistors of 2 Ω, 3 Ω and 5 Ω are connected in series. Calculate the total resistance. [1]

…………………………………………………….

…………………………………………………….

8. [Supplement] A 6 V battery powers two resistors in series. The p.d. across the first resistor is 2 V. Calculate the p.d. across the second resistor. [1]

…………………………………………………….

…………………………………………………….

9. [Supplement] Two resistors of 6 Ω and 12 Ω are connected in parallel. Calculate the combined resistance. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

10. An appliance normally uses 7 A. Which fuse should be used: 3 A, 5 A, or 13 A? [1]

…………………………………………………….

…………………………………………………….

Section C: Analysis Questions

11. A circuit contains a 12 V battery and two lamps in parallel. Each lamp has a resistance of 6 Ω.

(a) What is the p.d. across each lamp? [1]

(b) What is the combined resistance of the two lamps? [2]

(c) Calculate the current from the battery. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

12. [Supplement] A potential divider consists of a 4 kΩ resistor (R₁) and a 6 kΩ resistor (R₂) connected in series to a 10 V supply.

(a) Calculate the p.d. across R₁. [2]

(b) Calculate the p.d. across R₂. [1]

(c) If R₂ is replaced by an LDR, explain what happens to V_out when light intensity increases. [2]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

13. Explain why lighting circuits in homes use lamps connected in parallel rather than in series. [3]

…………………………………………………….

…………………………………………………….

…………………………………………………….

…………………………………………………….

Exercise 1

| 1. | Which component has a resistance that decreases as temperature increases? |  |
| --- | --- | --- |
|  | A | LDR |
|  | B | Thermistor (NTC) |
|  | C | Fixed resistor |
|  | D | Diode |

| 2. | An ammeter must be connected: |  |
| --- | --- | --- |
|  | A | In parallel with the component |
|  | B | In series with the component |
|  | C | Across the battery |
|  | D | In any position |

| 3. | Three 1.5 V cells are connected in series. What is the total e.m.f.? |  |
| --- | --- | --- |
|  | A | 1.5 V |
|  | B | 3.0 V |
|  | C | 4.5 V |
|  | D | 6.0 V |

| 4. | In a series circuit, the current: |  |
| --- | --- | --- |
|  | A | Is largest near the battery |
|  | B | Is smallest near the battery |
|  | C | Is the same at every point |
|  | D | Decreases as it passes through each component |

| 5. | Two resistors of 4 Ω and 6 Ω are connected in series. What is the total resistance? |  |
| --- | --- | --- |
|  | A | 2.4 Ω |
|  | B | 4 Ω |
|  | C | 6 Ω |
|  | D | 10 Ω |

| 6. | In a parallel circuit, the p.d. across each branch is: |  |
| --- | --- | --- |
|  | A | Different for each branch |
|  | B | The same for each branch |
|  | C | Zero |
|  | D | Half the supply voltage |

| 7. | Two 10 Ω resistors are connected in parallel. The combined resistance is: |  |
| --- | --- | --- |
|  | A | 20 Ω |
|  | B | 10 Ω |
|  | C | 5 Ω |
|  | D | 0 Ω |

| 8. | The live wire in a UK mains plug is coloured: |  |
| --- | --- | --- |
|  | A | Blue |
|  | B | Green and yellow |
|  | C | Brown |
|  | D | Red |

| 9. | A fuse is designed to: |  |
| --- | --- | --- |
|  | A | Increase the current in a circuit |
|  | B | Store electrical energy |
|  | C | Melt and break the circuit if the current is too high |
|  | D | Reduce the voltage |

| 10. | A double-insulated appliance does not need an earth wire because: |  |
| --- | --- | --- |
|  | A | It uses very low voltage |
|  | B | Its outer case is non-conducting |
|  | C | It has a fuse instead |
|  | D | The neutral wire provides protection |

Self-Assessment Checklist

After studying this module, I can:

Section 17.1 – Circuit Diagrams & Components:

□  Draw and interpret circuit diagrams using standard symbols

□  Describe how thermistors and LDRs behave with temperature/light changes

□  [Supplement] Explain how diodes allow current in one direction only

Section 17.2 – Series Circuits:

□  State that current is the same at all points in a series circuit

□  Calculate combined e.m.f. of cells in series

□  [Supplement] Calculate missing p.d. values using E = V₁ + V₂ + …

□  Calculate combined resistance for resistors in series

Section 17.3 – Parallel Circuits:

□  State that current from source is larger than current in each branch

□  [Supplement] Use the junction rule: I_in = I_out

□  [Supplement] State that p.d. is the same across each branch

□  State that combined resistance in parallel is less than either resistor

□  [Supplement] Calculate combined resistance using 1/R = 1/R₁ + 1/R₂

□  Explain advantages of parallel lighting circuits

Section 17.4 – Potential Dividers:

□  Relate p.d. to resistance for constant current

□  [Supplement] Describe how a potential divider works

□  [Supplement] Use the equation R₁/R₂ = V₁/V₂

Section 17.5 – Electrical Safety:

□  State common electrical hazards

□  Name the three mains wires and their colours

□  Explain why the switch connects to the live wire

□  Explain how fuses and trip switches protect circuits

□  Explain earthing and double insulation